Physicochemical study of nitrides, oxynitrides and oxides of transition metals for water photoelectrolysis

Alfredo Giner Requena

Institut Universitari d’Electroquímica i Departament de Química Física, Universitat
d’Alacant, Apartat 99, 03080 Alacant (Spain)

alfredo.giner@ua.es

Mentor: Roberto Gómez Torregrosa. Universidad de Alicante

Director: Teresa Lana Villarreal. Universidad de Alicante

After years of studies in search of materials able to perform water photoelectrolysis, the holly grail material has not been found. The ideal material should fulfill the next criteria: to be environmentally abundant, sustainable and friendly, to have a band gap that allows absorption of a large part of the solar spectrum, to have suitable energy band edges to carry out the corresponding reduction and oxidation reactions of water, to show long-term stability against (photo)corrosion and to show fast kinetics for the photogenerated charge carriers transfer to the species in the electrolyte with a low recombination rate.

In the context of this PhD, our aim is to study nitrides and oxynitrides of transition metals. The interest is that the hybridization of the O 2p and N 2p orbitals, leading to the valence band, induces a decrease in the band gap of the material. ¹ Currently the number of studies is limited. The most outstanding reported materials are Ta₃N₅, TaON, BaTaO₂N, LaTiO₂N, or Ga and Zn oxynitrides, among others. 2 Eventually, ternary transition metal oxides from the perovskite and spinel families will be also studied. The number of possible candidates of ternary oxides is huge (more than 200,000). 3 This fact shows the large potential of these materials but also, the need of a guided selection.

The materials will be synthesized as thin films by sol-gel techniques or sputtering or methods based on the prior preparation of powdered particles. The as prepared films will be modified with co-catalysts, extracting layers or dopants to improve their photoelectrochemical behavior. The prepared materials will be studied and characterized: (i) morphologically (TEM, SEM, AFM), (ii) structurally (XRD, Raman Spectroscopy), (iii) compositionally (XPS, elemental analysis, EDX), (iv) optically (UV-Vis, TAS (Transitient Absorption Spectroscopy), Fluorescence Spectroscopy) and (v) photoelectrochemically (using techniques such as cyclic and linear voltammetry (with and without illumination) and impedance spectroscopy, among many others).

References

[1] ADHIKARI, Shiba P., et al. Visible light assisted photocatalytic hydrogen generation by Ta 2 O 5/Bi 2 O 3, TaON/Bi 2 O 3, and Ta 3 N 5/Bi 2 O 3 composites. RSC Advances, 2015, vol. 5, no 68, p. 54998-55005.

[2] ABEYSINGHE, Dileka; SKRABALAK, Sara E. Toward shape-controlled metal oxynitride and nitride particles for solar energy applications. ACS Energy Letters, 2018, vol. 3, no 6, p. 1331-1344.

[3] C. Yuan, H. Bin Wu, Y. Xie, X. W. D. Lou, Angew Chem. Int. Ed., 53,(2014) 1488.