Thermo-catalytic production of green hydrogen from biogas. Design and preparation of efficient catalysts.

Judit Lloreda Rodesa,b *

Mentor: Elvira Gómez

Directors: Elvira Gómez, Albert Serrà

a Consorci per a la Gestió dels Residus del Vallès Oriental (CGRVO), Camí Ral s/n, E-08401, Barcelona

b Grup d’Electrodeposició de Capes Primes i Nanoestructures (GE-CPN), Departament de Ciència

de Materials i Química Física, Universitat de Barcelona, Martí i Franquès, 1, E-08028, Barcelona

*juditlloreda@ub.edu

Abstract

In the current situation of climate emergency, renewable gases such as biogas, biomethane and hydrogen, stand out as new sources of green energy. Every year large amounts of biogas (CH4 and CO2) are produced from organic urban waste, and its reforming into green hydrogen stands as a way of valorisation. However, this conversion is still awaiting the development of new technologies that allow to reduce the high energy requirements of the process, while increasing the efficiency of H2 production [1]. Catalyst reduce the activation energy of a reaction, and bimetallic materials stand out as heterogeneous catalysts, specifically those based on nickel [2]. However, an external supply of energy will continue to be essential to reach high temperatures. In this context, a ferromagnetic support allows the innovative use of magnetic induction as this energetic supplier.

In this Thesis the focus is set on the electrochemical preparation of nickel-based catalysts combined with other metals like Co, Fe, Zn, … to produce green hydrogen by dry-reforming of biogas. For every alloy proposed, an electrochemical bath will be designed and studied on a glassy carbon substrate. Deposits will be prepared and characterized to assess the electrodeposition conditions upon which the electrochemical, morphological and structural characterization will be performed.

Catalysts for biogas reforming were prepared on a commercial porous stainless-steel substrate by depositing the alloy under different electrochemical conditions. The use of a ferromagnetic material is related to the proposal of the innovative use of magnetic induction as energy supply technique.

The catalytic performance of the prepared deposits will be firstly assessed using a conventional thermal reactor from which material and reaction parameters will be optimized. Finally, under those optimized reaction conditions, magnetic induction will be integrated as energy supplied technique, by adjusting the necessary magnetic parameters to reach the reaction temperature.

Acknowledgments

With the support of the Industrial Doctorates Program from AGAUR-Generalitat de Catalunya (project 2022 DI 035). J.LL. acknowledges the support of Dra. Vanessa Abad from CGRVO.

References

[1] Jung, S., Lee, J., Moon, D.H., Kim, K.H., E. Kwon, E.E., Renewable and Sustainable Energy Reviews 2021, 143, 110949-110960

[2] Fazlikeshteli, S., Vendrell, X., Llorca, J. 2023. Low-temperature partial oxidation of methane over Pd-Ni bimetallic catalysts supported on CeO2. Int. J. Hydrogen Energy 48, 12024-12035.